Anas Siddiqui

EET 418

Lab 12.1

;Volume of box=x*y*z

DATA SEGMENT

X   DW  0ABh

Y   DW  0CDh

Z   DW  0EFh

DATA ENDS

CODE SEGMENT

ASSUME DS:DATA,CS:CODE

VOLUME:   MOV AX,DATA ; MOVE DATA TO AX REGISTER

MOV DS,AX  	 ; MOVE AX TO DATA SEGMENT

XOR AX,AX   	; CLEAR AX REGISTER

MOV AX,X    	; MOVE X TO AX

XOR BX,BX   	; CLEAR BX REGISTER

MOV BX,Y    	; MOVE Y TO BX

MUL BX      	; MULTIPLY AX TO BX

MOV BX,Z   	 ; MOVE Z TO BX

MUL BX      	; MULTIPLY AX TO BX

HLT

CODE ENDS

END VOLUME

12.2

;Volume of box=x*y*z

DATA SEGMENT

X   DW  0ABCDh

Y   DW  0BCDEh

Z   DW  0CDEFh

DATA ENDS

CODE SEGMENT

ASSUME DS:DATA,CS:CODE

VOLUME:   MOV AX,DATA ; MOVE DATA TO AX REGISTER

MOV DS,AX   ; MOVE AX TO DATA SEGMENT

XOR AX,AX   ; CLEAR AX REGISTER

MOV AX,X    ; MOVE X TO AX

XOR BX,BX   ; CLEAR BX REGISTER

MOV BX,Y    ; MOVE Y TO BX

MUL BX      ; MULTIPLY BX BY AX

MOV CX,DX   ; MOVE DX TO CX

MOV BX,Z    ; MOVE Z TO BX

MUL BX      ; MULTIPLY BX TO AX

MOV DI,AX   ; MOVE AX TO DI

MOV AX,CX   ; MOVE CX TO AX

MOV CX,DX   ; MOVE DX TO CX

MUL BX      ; MULTIPLY BX BY AX

ADD AX,CX   ; ADD AX AND CX

MOV SI,AX   ; MOVE AX TO SI

MOV BP,DX   ; MOV DX TO BP

HLT

; THE ANSWER WILL BE IN BP,SI,DI BECAUSE

; IT WILL NOT FIT IN 16 BIT REGISTER

CODE ENDS

END VOLUME