K = VoVi

db= 20logk           ∴k=10db20

R1 = R0 1-k1+k

R2 = R02-R122R1 = R0 2K1-K2

Assignment:

- Use MATLAB (and symbolic Math) to determine the design equations for R1 and R2 in terms of R0 and k.

Simplify the MATLAB results!

Submit the __.m file and the results.

- Verify that a T – section attenuator with resistors R1= 21.52527 AND R2= 47.31 ohms will produce 8-dbs of attenuation.

Draw the circuit and show all the calculations.

R1+R2            -R2-R2            R1+R2 I1I2 = Vin0

I2= R1+R2            Vin-R2                   0R1+R2            -R2-R2            R1+R2  = -R2Vin(R1+R2+RL)(R1+R2) – R22

V-out = I2*RL = -R2VinRLR1+R2+RL)(R1+R2) – R22

k= VoVi = -R2RL (R1+R2+RL)(R1+R2) – R22

R1= 21.52527 ohms

R2= 47.31 ohms

k = -21.52527 47.31 (-21.52527+47.31+50)(-21.52527+47.31) – 47.312 ≈ - 0.3955

k= VoVi ≈- 0.396

20log1/0.396≈ 8.06 db

kdb≈8.06 db