Lab#9:

T(s)=  25 / s^2 + 6s + 25

| Parameters | Peak Time (Tp) | Settling Time (Ts) | Overshoot % |
| --- | --- | --- | --- |
| Theoretical Results | 0.78 s | 1.33 s | 9.44 |
| Experimental Results Original T(s) | 0.80 s | 1.50 s | 10.0 |
| Experimental Results Add. Zero=3 | 0.40 s | 1.6 s | 40.0 |
| Experimental Results Add. Zero=15 | 0.70 s | 1.5 s | 10.0 |
| Experimental Results Add. Zero=3 and pole =6 | 0.60 s | 1.7 s | 22.0 |
| Experimental Results Add. Zero=15 and pole =6 | 1.1 s | 2.0 s | 10.0 |

Conclusion:  The objective of this lab is to examine the effect of a real “zero” on the step response of a second order under damped system. We will also examine the step response of the system when a real “zero” and a third real “pole” are present simultaneously. Certain parameters will be observed in order to determine the effect of the systems for each case according. These parameters are the % overshoot, peak time and settling time. The original transfer function was plotted using Mathlab and then compared to the estimated values to the theoretical values as seen in the table above. After  the original transfer function was plotted, various “zero” was added to the transfer function and the step response was obtained. The peak time, settling  time,  and percent overshoot was then  to various “zero” added to the transfer. From the table above, the greater the “zero” value, the closer the estimated value were closer to the original function.